Safe Work Method Statement

Organisational Details

| Business Undertaking the Work: |  | A.B.N: |  |
| --- | --- | --- | --- |

| Approved for Use By: |  | Signature: |  |
| --- | --- | --- | --- |

| Person Overseeing the SWMS: |  | Contact Number: |  |
| --- | --- | --- | --- |

Project and Principal Contractor Details

| Scope of the Work: | Installing Ceiling Joists |
| --- | --- |

| Project Address: |  |
| --- | --- |

| Principal Contractor (P.C.): | Loreco Pty Ltd |  |
| --- | --- | --- |

| P.C. Contact Person: | Matt Westle | Contact Number: | 0447122611 |
| --- | --- | --- | --- |

| Approved for Use By: | Matt Westle | Signature: | MW |
| --- | --- | --- | --- |

SWMS Details

| SWMS Developed By: | M Westle | Contact Number: | 0447122611 |
| --- | --- | --- | --- |

| Title ¦ Position: | M Westle | Date Developed: | 1.12.17 |
| --- | --- | --- | --- |

| Review Date: |  | 19.07.19 |  |  |  |  |  | (12 months max) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Monitoring and Review: |  | Visual monitoring of control measures will be undertaken and reviewed if circumstances change. The SWMS will be amended if there is a change in the activity. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consultation: |  | Relevant personnel (including HSR’s where established) have been consulted in the development, and where required, review and amending of this SWMS. |  |  |  |  |  |  |  |  |  | ☐ |  | Yes | ☐ | No |
| High Risk Construction Work Associated with this SWMS |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  | YES | ☐ | NO |
| If YES, High Risk Construction Work Involving: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☒ | The risk of a person falling 2.0 meters or more |  |  |  |  | ☐ | Or is likely to involve, the removal or likely disturbance of asbestos |  |  |  |  |  |  |  |  |  |
| ☐ | Demolition of an element of a structure that is load-bearing or otherwise related to the physical integrity of the structure |  |  |  |  | ☐ | Work in, on or near an area at a workplace in which there is any movement of powered mobile plant |  |  |  |  |  |  |  |  |  |
| ☐ | Work in or near a shaft or trench with an excavated depth greater than 1.5m or a tunnel |  |  |  |  | ☐ | Structural alterations or repairs that require temporary support to prevent collapse |  |  |  |  |  |  |  |  |  |
| ☐ | Work in, on or near an area that may have a contaminated or flammable atmosphere |  |  |  |  | ☐ | Work in, on or adjacent to a road, railway, shipping lane or other traffic corridor that is in use by traffic other than pedestrians |  |  |  |  |  |  |  |  |  |
| ☒ | Work in, on or near energised electrical installations or services |  |  |  |  | ☐ | Tilt-up or precast concrete |  |  |  |  |  |  |  |  |  |
| ☐ | Work in, on or near an area in which there are artificial extremes of temperature |  |  |  |  | ☐ | Work in, on or near water or other liquid that involves a risk of drowning |  |  |  |  |  |  |  |  |  |
| ☐ | Work in, on or near chemical, fuel or refrigerant lines |  |  |  |  | ☐ | Work in, on or near a confined space |  |  |  |  |  |  |  |  |  |
| ☐ | Work in, on or near pressurised gas distribution mains or piping |  |  |  |  | ☐ | Work on a telecommunications tower |  |  |  |  |  |  |  |  |  |
| ☐ | Diving work |  |  |  |  | ☐ | The use of explosives |  |  |  |  |  |  |  |  |  |
| Other Hazards / Considerations Associated with this SWMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ | Access ¦ Egress |  | ☐ | Contaminated Landfill |  | ☐ | Biological ¦ Bacterial |  | ☐ |  |  |  | Scaffolding |  |  |  |
| ☐ | Crushing ¦ Entrapment |  | ☐ | Waste Management |  | ☒ | Manual ¦ Materials Handling |  | ☐ |  |  |  | Signage |  |  |  |
| ☐ | Demolition |  | ☐ | Hot Work |  | ☐ | Structural Alterations / Support |  | ☐ |  |  |  | Fatigue |  |  |  |
| ☐ | Explosive Power Tools |  | ☐ | Lighting |  | ☒ | Electrical Energy |  | ☐ |  |  |  | Fire ¦ Explosion |  |  |  |
| ☐ | Firearms |  | ☐ | Emergency Response |  | ☒ | Energy Sources (other than electrical) |  | ☐ |  |  |  | Fire Protection |  |  |  |
| ☒ | Fumes ¦ Dust ¦ Steam |  | ☐ | Plant and Equipment |  | ☐ | Hazardous Chemicals / Substances |  | ☒ |  |  |  | Existing Services |  |  |  |
| ☒ | Flying ¦ Falling Objects |  | ☐ | Noise |  | ☐ | Dangerous Goods |  | ☐ |  |  |  | Traffic Management |  |  |  |
| ☐ | Lasers |  | ☒ | Public ¦ Occupants ¦ People |  | ☐ | Lead |  | ☒ |  |  |  | Ventilation |  |  |  |
| ☐ | Working Alone ¦ Isolation |  | ☐ | Young ¦ Inexperienced Workers |  | ☐ | Synthetic Mineral Fibres |  | ☐ |  |  |  | Flora / Fauna |  |  |  |
| ☒ | Slips ¦ Trips ¦ Falls |  | ☐ | Trenching ¦ Excavations |  | ☒ | Machine ¦ Equipment Guarding |  | ☐ |  |  |  | Working Environment |  |  |  |
| ☐ | Formwork ¦ Falsework |  | ☒ | Housekeeping ¦ Storage |  | ☒ | Working at Height ¦ Edge Protection |  | ☐ |  |  |  | Climatic Conditions |  |  |  |
| ☐ | Design ¦ Overloading |  | ☐ | Visibility |  | ☐ | Animals ¦ Insects |  | ☒ |  |  |  | Training and Induction |  |  |  |

Supplementary Information

Plant | Equipment Involved

| - Mobile Scaffold ¦ Modular Scaffold ¦ Trestle Scaffold | - Portable RCD |
| --- | --- |
| - Step / Extension Ladder | - Extension Leads |
| - Nailing Gun | - Circular Saw |
| - Air Compressor |  |

Qualifications | Certificates of Competency | Experience | Training | High Risk Licences

| - Basic Scaffolding (SB) | - Construction Induction Training (White Card) |
| --- | --- |

Reference Relevant Legislation | Codes of Practice / Compliance Codes | Australian Standards | Safety Data Sheets (S.D.S.)

Engineering Details | Certificates | Approvals

Personal Protective Equipment (PPE) Requirements

☒ ☒ ☒ ☐ ☒ ☐ ☐ ☐ ☐ ☒ ☒ ☐ ☒

Other ¦ Additional PPE Requirements

| No | Task ¦ Activity | # Potential Hazards and Risks | Control Measures |
| --- | --- | --- | --- |
| 1 | Training | - Lack of training leads to increased risks | - All workers are experienced and skilled in the task of installing ceiling joists (single storey) - All workers must be trained in this SWMS and how to use the control measures as detailed. |
| 2 | Working Platforms on Scaffolds | - Fall from height - Falling objects | - No scaffold to be erected within the “No Go Zone” for overhead power lines. - All scaffold work platforms over 2 metres high to have fall protection. - Scaffold over 4 metres high is only to be erected / modified and dismantled by a person with a scaffold certificate. - Scaffolds to comply with relevant parts of the Australian Standard for Scaffolding. - Scaffold erection is to be done in accordance with suppliers instructions, is as near as reasonable to level, and is fully decked when being used. - Where the scaffold is to be loaded, the scaffolding is to be capable of supporting a heavy duty live loading (approx. 650kg per bay) |
| 3 | Mobile Scaffolds | - Fall from height - Falling objects | - Where mobile scaffolds are used, ensure that they are built to standard as per the manufacturers specifications, erected on flat hard surfaces, no higher than 3 times the minimum base dimension. |
| 4 | Working Platforms on Adjustable Trestles | - Fall from height - Falling objects | - Platforms (eg scaffold planks) on trestles provide a bigger, more stable surface to work from than a ladder rung. - Trestles are to be erected in accordance with the supplier’s instructions, and good practice, as outlined in the AS 1576.5 for adjustable trestles and AS 4576. - Where the platform or trestles is to be loaded with bricks, the trestles and platforms are capable of supporting a heavy duty live loading of about 650kg per bay - for example 120 bricks, mud board and 2 persons, as outlined in AS 1576 & AS 4576 - Platforms are to be erected as near as reasonable to level, and are stepped rather than sloped. - Platforms are in good sound condition and planked for heavy duty work. - Make sure attention is given to placing trestles on a firm surface. |
| 5 | Working at Heights | - Fall from height | Workers are Not to Walk the External Top Plate - Appropriate portable ladders and temporary work platforms (i.e. trestle scaffolds) will be used when working at heights up to and greater than 2 meters, otherwise a risk assessment may determine the need for additional risk control measures. - All stairwells, atriums and voids should be sturdily guarded, covered with an industrial safety net or sheeted over, regardless of the fall distance from the upper level floor. |
| 6 | Manual Handling | - Muscle strain | - When lifting awkward and heavy items, use mechanical aids. Where this is not possible consider team lifting where people are trained and encourage and use good manual handling techniques: - When bending to lift objects maintain a straight back & bent knees to a semi-squat where your feet are stable on the ground & assume a power lift stance. |
| 7 | Fabrication and Fixing of Ceiling Joists | - Electrocution - Puncture wound from nailing gun - Compressed air - Cuts while using power saws - Eye Injury - Noise - Dust | - All leads in use to be tested and tagged at 3 month intervals and prior to use. - All leads kept of the ground and to be connected to a G.P.O or portable device protected by a Residual Current Device (RCD). The RCD to be tested every month. - Nail guns to be use by only trained and competent persons. - Keep body parts clear of the firing line and do not squeeze trigger until the nail gun is in position on the timber. - Keep others away from the firing zone. - Guards to be fitted and working effectively, adequately maintained and used in accordance with the manufactures instructions. - Keep body parts away from the cutting line. - Minimise noise by appropriate maintenance of tools and use of notched cutting blades. - Operator to wear impact resistant eye wear. - Keep others away as far as possible. - Operators to use appropriate ear muffs or ear plugs. - If unventilated area wear suitable dust mask. |
| 8 | Housekeeping | - Tripping over material - Standing on exposed nails | - Keep areas clean and clear. - Place all waste material in bins. - All timber to be de nailed or any nails to or any nails to be bent over to prevent injury. |

Persons Involved in the Activity

I have read, understood and will comply with the requirements of this Safe Work Method Statement

| Name | Company ¦ Employer | Signature | Date |
| --- | --- | --- | --- |

Site Specific Hazards

List here any site specific hazards & risks (including control measures) that are additional to this SWMS

| No | Task ¦ Activity | # Potential Hazards and Risks | Control Measures |
| --- | --- | --- | --- |